Practical-3

Name:- Shubham Sonawane

Roll no-4401058

Div-A   Batch-D

3rd experiment (yash cha Ultrasonic sensors wala)

#define BLYNK_TEMPLATE_ID "TMPL36jrZnpQh"

#define BLYNK_TEMPLATE_NAME "Interfacing Sensors And Actuators With ESP8266"

#define BLYNK_AUTH_TOKEN "b_aZa5wq5V5MUnIa4ad9M6RX698hIA33"

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#include <LiquidCrystal_I2C.h>

// --- WiFi Credentials ---

char ssid[] = "OnePlus 11R 5G";

char pass[] = "YBorse5029";

// --- LCD Setup (I2C) ---

LiquidCrystal_I2C lcd(0x27, 16, 2); // Adjust address if needed

// --- Pin Configuration ---

#define trigPin D5

#define echoPin D6

#define irPin D7

#define tempPin A0

// --- Variables ---

long duration;

int distance;

int irValue;

float temperature;

// --- Timer for Blynk ---

BlynkTimer timer;

// --- Function to read sensors and update LCD/Blynk ---

void readSensors() {

// --- IR Sensor ---

int rawIR = digitalRead(irPin); // LOW = object detected (depends on module)

if(rawIR == LOW) irValue = 1; // Object detected

else irValue = 0; // No object

Blynk.virtualWrite(V2, irValue);

// --- Ultrasonic Sensor ---

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH, 30000); // 30ms timeout

if(duration == 0) distance = 0; // No echo detected or object too close

else distance = duration * 0.034 / 2; // Convert to cm

// Limit max distance to avoid showing huge numbers

if(distance > 400) distance = 400;

Blynk.virtualWrite(V1, distance);

// --- Temperature Sensor (LM35) ---

int adcValue = analogRead(tempPin);

float voltage = adcValue * (3.3 / 1023.0); // ESP8266 ADC ref = 3.3V

temperature = voltage * 100; // LM35: 10mV per °C

Blynk.virtualWrite(V0, temperature);

// --- Update LCD Display ---

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("D:");

lcd.print(distance);

lcd.print("cm ");

lcd.print("IR:");

lcd.print(irValue);

lcd.setCursor(0, 1);

lcd.print("T:");

lcd.print(temperature);

lcd.print("C");

// Optional: Send LCD text to Blynk (V3)

String lcdData = "D:" + String(distance) + "cm IR:" + String(irValue) + " T:" + String(temperature) + "C";

Blynk.virtualWrite(V3, lcdData);

}

void setup() {

Serial.begin(9600);

// --- LCD Init ---

lcd.init();

lcd.backlight();

// --- Pin Modes ---

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(irPin, INPUT);

// --- Connect to Blynk ---

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// --- Run readSensors() every 1 second ---

timer.setInterval(1000L, readSensors);

}

void loop() {

Blynk.run();

timer.run();

}

OUTPUT:

Blynk IOT Dashboard -

Hardware Output: