Practical-7 Bluetooth

🔹 Execution Steps

- Open Arduino IDE on your computer.

- Connect your Arduino UNO via USB.

- Paste this code into a new sketch:

#include <SoftwareSerial.h>

SoftwareSerial BTSerial(2, 3); // RX, TX

void setup() {

Serial.begin(9600);

BTSerial.begin(9600);

Serial.println("Bluetooth ready");

}

void loop() {

if (BTSerial.available())

Serial.write(BTSerial.read());

if (Serial.available())

BTSerial.write(Serial.read());

}

NOTE:  CODE IS PRESENT IN THE ARDUINO IDE ALREADY

GO TO File → Open → choose “Exp_7”

- Upload the code to your UNO (Ctrl + U).

- Open Serial Monitor (Ctrl + Shift + M):

  - Baud rate: 9600

  - Line ending: No line ending

- Power your HC-05 (it should already be wired to the UNO).

  - LED should blink fast (2 blinks per second) = normal mode.

- On your Android phone:

  - Open Serial Bluetooth Terminal app.

  - Go to Settings → Devices → “HC-05” → tap Connect.

  - The LED on the module will change to slow double blink or steady on (connected).

- Test communication:

  - In the phone app, type a message → press send → you’ll see it appear in Arduino Serial Monitor.

  - In Arduino Serial Monitor, type a message → press send → it appears instantly in the phone app.

OUTPUT:

HARDWARE

SERIAL TERMINAL

SERIAL MONITOR